# Name: __________	Class: ___	Date: ________

# Model 3 - Windscreen wipers - Parallel

## Construction task

In this version, the wiper blades are always vertical and are guided across the windscreen in this position, too. You can see this on the large windscreens of buses, for example.

Construct the model according to the building instructions. Please note that the two cranks for the wiper arms are pointing downwards in this model.

## Topic task

In the “rest position” the two wiper blades should both be at the outside edge (in the centre they would restrict the view too much). The two eccentrics now have to be adjusted in two respects:

- Adjust the eccentricity (the distance between the axis of rotation of an eccentric and the attachment point of the I-strut 45 on this) in such a way that a suitable swivel range is produced. The wiping ranges of both wipers should overlap so that the entire windscreen is really wiped. Thus the eccentricity must be large enough for the wipers to move somewhat beyond the middle of the 9 holes in the horizontal structural girder between the wipers. This brings us to the second adjustment:

- When they are in the middle, the wipers must not get in each others way and collide. Otherwise the mechanism would be blocked. This means that one of the wipers has to start moving from the outside to the inside somewhat earlier than the other. For this, there is a separate eccentric for each wiper arm in this model. We achieve the different starting point of the wipers by attaching the eccentrics somewhat offset to one another:

- In terms of design, you can achieve this if you pull one of the gear wheels Z20 off a little and attach it offset by one or more teeth. This results in the eccentrics not being at the same angle as the rest of the model at the start. This is known as advancing (in the case of the eccentric which has been turned somewhat further) or lagging (with the eccentric which is a little way behind).

- If we would like the wiper on the driver’s side to start first (because this is the more important side for the driver): Which of the two eccentrics then has to advance and which to lag?

- __________________________________________________

__________________________________________________

## Experimental task

- Play with the magnitude of advance/lag between the two eccentrics. How many teeth offset between the two Z20 gear wheels results in the model working well?

- __________________________________________________

- What happens if you make the offset too big and what is unfavourable about that?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

# Attachments

## Windscreen wipers - Parallel

# Further information

[1]	Wikipedia: Parallelogram guidance.

[2]	fischerwerke: Vor- und Nacheilung (Advance and lag). In: fischertechnik Experimente+Modelle, hobby 1 Band 2, 1973. In the fischertechnik database (PDF, direct link), p. 50-53.