# Name: __________	Class: ___	Date: ________

# Model 5 - Press - Toggle press

## Construction task

A toggle mechanism works in exactly the same way as the knee joint in a human leg. Let us take a very simplified look at a leg and a cyclist’s pedal:

The thigh is green, the lower leg orange and the track described by the pedal is shown as a blue circle. On the left of the illustration we see the leg at an angle, because the pedal is a the top. On the right, the leg is extended almost completely. Not for nothing! If you use muscular power to extend your leg, it can exert the most downward force on the pedal at this point.

Now let’s imagine two joint-mounted and joint-linked levers instead of the leg. At the top and bottom the joints correspond to the hip and the foot, the middle joint is the knee. Now let’s imagine that these levers are not moved by “muscle power” but rather by pressure from the front on the knee itself.

This is indicated by the red arrow on the second drawing:

In the position shown on the right of the illustration, the pressure on the knee in the direction of the arrow exerts a particularly large downward force on the foot (providing the upper end of the upper lever - the “hip” – is fixed). This is the trick of how you can use only a little force (red arrows) to get a lot of force (but with slower motion, grey arrows).

We want to try this out. Build the model according to the building instructions. Make sure that the vertically upright axles are adjusted in such a way that the press head can move smoothly up and down.

## Topic task

- Describe the three components (assemblies) of the model through which the rotational force at the crank runs to the pressing force in the press head:

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- Compare a) the effect of the combination of eccentric crank and the lever attached to it (the I-strut 30) with b) the effect of the two levers in the toggle mechanism. What is different, what is the same?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

## Experimental task

Place objects of different thicknesses under the press, such as paper, putty, thin pieces of wood.

- Assuming the “housing” of the press holds, is the maximum force of this toggle press model less than, equal to or greater than that of the simple eccentric press?

- __________________________________________________

__________________________________________________

# Attachments

## Press - Toggle press

# Further information

[1]	Wikipedia: Toggle.

[2]	Wikipedia: Toggle press.