# Name: __________	Class: ___	Date: ________

# Model 6 - Toggle joint with over-extension - Aircraft landing gear

## Construction task

The toggle joint from the previous model 05 (toggle joint press) is extended by over-extension and locking – demonstrated using the example of an aircraft landing gear.

The toggle joint we got to know in the toggle joint press has another application: If you move it slightly beyond the completely extended state (“over-extended”) and then simply prevent it from “buckling” under pressure due to the geometry of the structure - by “locking” it - you can use it to extend something that will not retract without further aids or controls (which could, after all, fail), even in the face of large external forces.

The illustration shows the non-extended state on the left and the completely extended state in the middle. The toggle joint on the right of the drawing is slightly over-extended. The element shown in light blue, which is fixed to the blue “lower leg” prevents the joint being over-extended further.

At the same time, it prevents the entire joint buckling. This way the mechanism can absorb very large forces (shown by the grey arrow). Protection against buckling - at least until material failure - is straightforward and safe thanks to a simple design.

One classic example of this application is for aircraft landing gear. Here, toggle joints with over-extension and locking are important elements for reliability and safety during starting and particularly during landing. Let’s have a look at this on the model.

Construct the model according to the building instructions.

- The upper horizontal beam stands for part of the aircraft body.

- The movable set of wheels stands for the nose landing gear, for example. It is to be retracted during the flight to improve the aircraft’s aerodynamics, and is only required for start, landing and when the plane is at a standstill.

- The supports made of angle girders only serve to hold the model and are not part of a real aircraft of course.

Check that by turning the crank the wheels of the landing gear can be retracted smoothly and extended all the way to the building plate 500.

## Topic task

Seen from the side with the crank, the aircraft is flying to the right. Extend the landing gear.

- Name two forces and their directions which act on the wheels when the aircraft touches down for landing.

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- Which of the girders to which the wheel assembly is fastened to the aircraft mainly absorb these forces?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- The landing gear is only “locked” when the toggle mechanism has been over-extended. What could happen during landing without locking and over-extension? For this, press onto the wheels “from the front” (in the direction of travel) while the landing gear is extended to different extents.

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

## Experimental task

To enable retraction and extension to be done without a great deal of effort, there is a multi-speed gear drive in the top of the structure.

- Turn the crank and try to find out how many revolutions of the crank you would need until the last gear wheel (the large Z30) rotates once?

- __________________________________________________

- There are gear wheels with 10, 20 and 30 teeth installed. A Z10 engages in a Z20, which in turn has a Z10 on its axle, which drives the Z30. What is

- 2010×3010

- and what can we learn from this?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________