# Name: __________	Class: ___	Date: ________

# Model 7 - Counter

## Construction task

Purely mechanical calculations could be made with the famous abacus [1] more than 2,000 years BC. Until relays, electron tubes, transistors and finally integrated circuits were introduced, there were no other tools for calculation than mechanical ones. However, this includes not only simple adding machines, but also those that could multiply, divide [2] [3] [4] and even calculate square roots [5], and of course the analogue slide rule [6], which was still widespread until the 1970s.

Our model implements a counter with three decimal places that can count from 0 to 999.

Build the model according to the building instructions. It is made up of the following components:

- An entrance in the form of a crank which can be turned.

- Three almost identically built counting stages, one each for the units, tens and hundreds place with the following elements:

  - A display (a ratchet gear wheel Z20 with glued-on scale), together with an angular block 60° whose tip indicates the current digit.

  - A detent lock, which ensures that the gear wheels stop at a defined position if they are not moved straight.

  - A carry-over mechanism to the next counting stage.

If the stickers with the digits have not been attached to the red gear wheels Z20 yet: Don’t attach the stickers with the digits yet!

Just build the model from the fischertechnik components. Adjust it in this order:

- Turn the crank clockwise and first, just observe which direction the red gear wheels Z20 revolve in.

- The tips of the angular blocks 60° are to mark the respective digit which indicates the current counter level. The gear wheel Z20 near to the crank is the units place, the middle one the tens and the last one the hundreds. There are two types of stickers.

- Question without digit stickers: If the machine is to count upwards from 000 to 999, which sticker has to be attached to which gear wheel?

- If they have already been attached, you can now observe which type of sticker has to be positioned on which gear wheel. If necessary, swap the gear wheels to have the right stickers in the right place on the counter.

- Adjust the model as follows, starting with the hundreds place:

- Set the last, individually installed gear wheel Z20 for the hundreds place in such a way that the detent below engages exactly in the space between two teeth.

- Set the middle Z20 for the tens place in such a way that its detent also engages exactly between two teeth and its carry-over tab (the red flat component) comes to rest straight over the two right-hand teeth of the hundreds Z20.

- Set the first Z20 for the units place in such a way that the detent under the Z10 engages exactly between two teeth and its carry-over tab rests straight under the two right-hand teeth of the tens Z20.

- Now you have to set the two carry-over tabs in terms of how far they engage in the teeth of the respective next Z20. Adjust the tabs in such a way that after one revolution of a Z20 (i.e. when switching back from 9 to 0), the respective next Z20 is reliably advanced by exactly two teeth. Check the function thoroughly and readjust if necessary until the mechanism works well.

- Then set the counter again as in a) to c). This is the “000” position. If the stickers were already attached, set the counter back to “000” according to a) to c).

- If the stickers are not already attached: Now attach the appropriate sticker (see construction task 2) to each Z20 so that the number 0 points exactly upwards and is thus "marked" by the tip of the respective angular block 60°.

## Topic task

- Turn the crank and check perfect counting function. How often do you have to turn the crank to count from 0 to 010?

- __________________________________________________

- Count all the way through once, until the counter indicates 000 again. How many turns of the crank do you need for this? Don’t just count – calculate please!

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- Providing you had enough building blocks, could you extend the counter to any number of places? Give reasons for your answer.

- __________________________________________________

__________________________________________________

__________________________________________________

## Experimental task

- How can you quickly reset the counter to 000 (or any number you like)? Write a set of instructions for this.

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- Try to count backwards with the counter. What do you find?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

# Attachments

## Counter

# Further information

[1]	Wikipedia: Abacus.

[2]	Wikipedia: Mechanical calculator.

[3]	David G. Hicks: Early Calculators. In The Museum of HP Calculators.

[4]	Rick Bensene: Bohn Contex Model 10 Mechanical Calculator. Example of a purely mechanical table-top calculator for the four basic arithmetic operations. In The Old Calculator Museum.

[5]	David G. Hicks: The Friden SRW. Mechanical calculator with square root function. In The Museum of HP Calculators.

[6]	Wikipedia: Slide rule.