# Name: __________	Class: ___	Date: ________

# Model 8 - Lift - Pawl

## Construction task

Construct the model according to the building instructions. This small lift is an example for many machines with manually operated (rope) winches.

When you crank the winch in a clockwise direction, the lift should move upwards. If it moves down instead, the rope has been wound on the wrong way. In this case, continue winding clockwise and allow the rope to wind onto the rope winch in the other direction.

Once the lift has arrived at the top, it must be secured against unintentional lowering - otherwise it would plummet downwards unchecked if no-one held the winch in place. This is what a pawl does. In the model, this is the pivot-mounted basic building block. You can lift it (then the crank wheel can be turned freely in both directions) or set it down on the gear wheel Z30. In this case, the crank can still be turned clockwise (and the lift moves upwards), but in the downward direction of the lift, the pawl – sometimes known as click for the sound it makes - blocks the Z30. This way, the lift is secured safely against unintentional falling.

The pawl’s own weight keeps it on the gear wheel - an important point for the reliability of the blocking device. The principle is very simple - where there are no complicated parts, they can’t fail either!

## Topic task

- Play with the model and try out how it works. Then write a set of instructions explaining what to do to move the lift up safely.

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- Then write a set of instructions explaining what to do to move the lift down safely again.

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

## Experimental task

- Find possible weaknesses in the design! What kind of things could happen to prevent the pawl moving properly and thus endangering safety? What could you do in each case to improve the model?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

# Attachments

## Link - Pawl

# Further information

[1]	Wikipedia: Ratchet.

[2]	Wikipedia: Rope winch.