# Name: __________	Class: ___	Date: ________

# Model 9 - Crane rope winch - Pulley (block and tackle)

This task sheet should always be done after task sheet 8 since it requires knowledge about how a pawl works.

## Construction task

- Crane and rope winch:

- Construct the crane with a load basket according to the building instructions but without the block and tackle (the crane rope just ends at the basket).

- Test the function of the moving lever to pull the crane rope up by moving the lever backwards and forwards. Topic task 1 can be worked on with the model in this state of construction.

- Install the pulley wheels in the block. Topic task 2 can be worked on in this state of construction.

- Add the force gauge with its spring. Hook the rope into the force gauge. The experimental task can now be done.

## Topic task

- In this model, there is a lever which engages in gear wheel Z30 via a pawl. It can be used to lift the rope “in batches”. Consider the design of this lever:

- Its axis of rotation is, at the same time, the axis of rotation of the rope drum itself. The axle with the rope drum goes through the middle of the jointed building block which has a hole in the middle for this purpose. This is a design trick with different application possibilities [1].

- In addition, the gear wheel has a crank handle. You can pull the rope up very quickly with this. The pawl is secured by a stop, to prevent it being flung away from the gear wheel teeth and flipping over at high speed. Compare this with the previous task sheet 08 – Lift - Pawl.

- This means we have two (blocking) pawls on the gear wheel - one at the lever, one at the return stop. One of these is always engaged with the gear wheel both while the rope is being pulled up and when the lever is being turned back for the next stroke.

- Describe the advantages and disadvantages of using the crank compared with the lever.

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- How could the design be modified to be able to pull at the lever with the same force but exert more rotational force on the crank? In other words, to make the lever “more smooth-running”?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- What would be the disadvantageous “price to pay” for this advantage?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- The block and tackle makes it possible to pull the rope using little force (via the winch) and yet still lift heavier loads. To achieve this, the block and tackle distributes the load to several pieces of the rope running in parallel. Due to the deflection pulleys, the rope still has to be pulled to lift the load by a certain height, but the tensile force required is reduced by the same factor. [3] [4] [5].

- Compare the tensile force required to lift the weight of the basket (with other objects inserted as a load if appropriate) when the rope runs through one, two or three deflection pulleys.

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- Why do you think the force gauge was built jointed?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

## Experimental task

The force gauge allows us to measure how the block and tackle works. Now hook the rope into the force gauge instead of into the rope drum. The load pulling on the rope now causes the spring to be compressed. The moving mark (the small round disc) shows the force in Newtons (symbol N) on the glued-on scale [6].

Such a spring has the welcome property that the deflection (the amount by which its length changes compared to the relaxed state) is proportional to the acting force. Double force = double deflection. For this reason, the simple set-up of our force gauge is sufficient to measure the force, even if it is not very accurate (we will be considering the subject of measuring accuracy in more detail on task sheet 13).

Repeat topic task 2 using the force gauge and observe the different deflection depending on the weight and particularly depending on the number of deflection pulleys in the block.

- As the load, use a weight that makes the force gauge deflect completely without the block and tackle. Note the force reading (deflection of the spring).

- __________________________________________________

__________________________________________________

- Then thread the crane rope around one, two or three block and tackle pulleys and repeat the measurement. Enter all the measured results in the table as follows:

| Block and tackle deflection pulleys | Distribution of the load to how many rope strands | Expected deflection | Reading of deflection in N |
| --- | --- | --- | --- |
| None (rope directly at the basket) | 1 | (as large as possible due to suitable weight, but still within the measuring range of the force gauge) |  |
| One (large deflection pulley at the hook) | 2 | 12 the deflection without block and tackle |  |
| Two (the small deflection pulley underneath the top of the crane in added) | 3 | 13 the deflection without block and tackle |  |
| Three (the second small deflection pulley at the hook added as well) | 4 | 14 the deflection without block and tackle |  |

# Attachments

## Crane rope winch - Pulley (block and tackle)

# Further information

[1]	Stefan Falk: Raffiniertes mit Achsen (Clever work with axles). ft:pedia 3/2013, p. 38–41.

[2]	Artur Fischer: Krane. fischertechnik hobby, Experimente und Modelle, hobby 2 Band 4 (Cranes. fischertechnik hobby, experiment and models, hobby 2 volume 4), Fischer-Werke, 1975.

[3]	Dirk Fox: Flaschenzug (Block and tackle). ft:pedia 2/2014, p. 4–10.

[4]	Wikipedia: Flaschenzug (Block and tackle).

[5]	Artur Fischer: Der Flaschenzug (The block and tackle). In: fischertechnik hobby, Experimente und Modelle, hobby 1 Band 1, (fischertechnik hobby, experiments and models, hobby 1 volume 1), Fischer-Werke, 1972, p. 40-43.

[6]	Wikipedia: Newton (unit).

[7]	Wikipedia: Spring constant.